Component Datasheets — Summary Reference

Pinout and power specs for key motors, microcontrollers, and sensors

This is a quick-reference summary compiled for planning purposes. Before wiring or ordering, pull the full datasheet PDF from the manufacturer's site for the exact part/revision you buy — pinouts and ratings can change between hardware revisions.

# Raspberry Pi 5 (8GB)

| Spec | Value |
| --- | --- |
| CPU | Broadcom BCM2712, quad-core Arm Cortex-A76 @ 2.4GHz |
| Power input | USB-C, 5V/5A (PD) — official Pi 5 supply strongly recommended |
| GPIO | Standard 40-pin header |
| Connectivity | Dual-band 802.11ac Wi-Fi, Bluetooth 5.0/BLE, Gigabit Ethernet |
| USB | 2x USB 3.0, 2x USB 2.0 |
| Storage | microSD slot; PCIe 2.0 x1 for NVMe via HAT |

Note the increased power draw vs. Pi 4 — under-powering the Pi 5 causes it to throttle CPU and disable some GPIO features, which will show up as flaky behavior in servo timing if you're not careful.

# PCA9685 — 16-Channel PWM/Servo Driver

| Spec | Value |
| --- | --- |
| Interface | I2C (address range 0x40-0x7F via solder jumpers) |
| Logic voltage | 3.3-5V (level-shifted, safe with Pi/ESP32 logic) |
| Output channels | 16, 12-bit PWM resolution |
| Servo power | Separate V+ terminal block — NOT powered from I2C logic pins; feed from the servo rail directly |
| Typical use here | Drives all body servos from a single I2C bus, addressed individually per channel |

# Standard Hobby Servo (MG996R-class)

| Spec | Value |
| --- | --- |
| Operating voltage | 4.8-7.2V |
| Stall torque | ~9-11 kg·cm at 6V (varies by clone/manufacturer — treat generic-brand figures as optimistic) |
| Rotation range | ~120-180° typical, servo-dependent |
| Control signal | Standard PWM, ~50Hz, 1-2ms pulse width |
| Wiring | 3-wire: signal (usually orange/yellow), V+ (red), ground (black/brown) |

# Smart Servo (Dynamixel-class, e.g. XM430 series)

| Spec | Value |
| --- | --- |
| Operating voltage | 11-14.8V typical (model-dependent — check exact part) |
| Control interface | TTL or RS-485 half-duplex serial bus, daisy-chainable |
| Feedback | Position, velocity, load, and temperature reported back over the bus |
| Why use it here | Feedback lets the control system detect a stalled/jammed joint instead of just commanding blind |

# ESP32 Dev Board

| Spec | Value |
| --- | --- |
| Power input | 5V via USB or VIN pin |
| Logic level | 3.3V GPIO — NOT 5V tolerant on most pins; check board variant |
| Interfaces | I2C, SPI, UART, plenty of GPIO for servo PWM or driving a PCA9685 |
| Role in this build | Real-time servo/sensor timing, offloaded from the Pi's non-real-time OS |

# Electret/MEMS Microphone Module

| Spec | Value |
| --- | --- |
| Interface | I2S (digital, e.g. INMP441) or USB — I2S preferred for direct Pi GPIO wiring |
| Power | 3.3V typical for I2S MEMS modules |
| Role in this build | Optional voice-trigger input for interactive responses |

# Where to Get the Real Datasheet

- Raspberry Pi 5: raspberrypi.com/documentation — official product brief and mechanical drawings

- PCA9685: NXP Semiconductors official datasheet (search "NXP PCA9685 datasheet")

- Servos: the specific manufacturer's product page — generic "MG996R" listings vary a lot between resellers, so match against the actual unit you bought

- ESP32: Espressif's official datasheet for the exact module (e.g. ESP32-WROOM-32)