Control Theory

How feedback loops (PID) help this figure move accurately and smoothly

# Why Feedback Control Matters Here

A servo told to "go to 90°" doesn't teleport there — it accelerates, moves, and settles, and mechanical load (an arm's weight, friction in a hinge) affects how well it tracks the commanded position. For simple hobby servos, this is handled internally by the servo's own small onboard controller. But for anything you're controlling in a closed loop yourself — like smoothing a jerky neck motion, or holding a stable eye position while the head moves — a PID controller is the standard tool.

# PID in Plain Terms

PID stands for Proportional, Integral, Derivative — three ways of reacting to the difference between where a joint is and where you want it to be (the "error").

| Term | What it does | Effect if too high | Effect if too low |
| --- | --- | --- | --- |
| P (Proportional) | Pushes harder the further off-target you are | Overshoot, oscillation | Sluggish, never quite reaches target |
| I (Integral) | Accumulates past error to eliminate steady-state offset | Slow oscillation, overshoot | Persistent small offset (e.g. arm settles slightly short) |
| D (Derivative) | Reacts to the rate of change of error, damping fast motion | Jittery response to noise | Overshoot, slow settling |

# Where This Build Actually Needs PID

- Neck/head stabilization if you want the head to hold a level gaze while other joints move — a small PID loop reading an IMU (accelerometer/gyro) can correct neck servo position in real time.

- Smart-servo (Dynamixel-class) arm joints, where the servo's internal firmware already runs PID and exposes tunable gains — you mostly need to tune, not implement, in this case.

- Eye tracking, if you add a camera and want the eyes to smoothly follow a face rather than snapping to a raw pixel coordinate — PID smooths the tracking error into a natural-looking motion.

# What Doesn't Need PID

Basic hobby servos moving to a fixed pose (mouth position from the audio envelope driver, a simple wave gesture) don't need an external loop — you're just sending target positions and letting the servo's internal control handle the rest. Don't over-engineer this: add closed-loop control only where you can point to a specific problem it solves (drift, instability, load-dependent inaccuracy).

# Basic Tuning Process

- Start with I and D at zero. Raise P until the joint responds promptly but starts to overshoot/oscillate, then back off ~20-30%.

- Add a small amount of D to damp any remaining oscillation.

- Add I only if you see a persistent small steady-state error that P and D alone don't remove — and add it sparingly, since too much causes slow "hunting" oscillation.

- Tune one joint in isolation before combining motions — coupled joints (e.g. shoulder + elbow moving together) make tuning much harder to reason about.

# Practical Note for This Project

Given the actuator choices in this build, most joints (arms, jaw, eyelids) are open-loop hobby servos where PID isn't necessary. Reserve PID effort for the neck/eye stabilization subsystem if you build it — that's the one place in this design where closed-loop control earns its complexity.