Source Code

Starter Python and C++ structure for reading sensors and driving motors

These are minimal skeletons to establish structure and wiring conventions — expect to expand error handling, calibration offsets, and safety checks before relying on them.

# 1. Pi-side: Python servo command wrapper

Sends high-level pose commands to the ESP32 over serial. Assumes a simple line-based protocol: "SET <channel> <angle>\n".

# servo_bridge.py

import serial

import time

class ServoBridge:

def __init__(self, port="/dev/ttyUSB0", baud=115200):

self.ser = serial.Serial(port, baud, timeout=1)

time.sleep(2)  # allow the microcontroller to reset after serial open

def set_angle(self, channel: int, angle: float):

angle = max(0, min(180, angle))  # clamp to safe range

cmd = f"SET {channel} {angle:.1f}\n"

self.ser.write(cmd.encode())

def set_pose(self, pose: dict):

# pose = {channel: angle, ...}

for channel, angle in pose.items():

self.set_angle(channel, angle)

time.sleep(0.005)  # small stagger avoids current spike from all servos moving at once

def close(self):

self.ser.close()

if __name__ == "__main__":

bridge = ServoBridge()

neutral_pose = {0: 90, 1: 90, 2: 90, 3: 90}  # neck, eyes, jaw, etc.

bridge.set_pose(neutral_pose)

bridge.close()

# 2. Pi-side: Python audio-triggered jaw envelope (software fallback)

If not using a dedicated hardware envelope-follower board, this reads TTS output audio and derives a jaw angle from amplitude.

# jaw_from_audio.py

import numpy as np

import sounddevice as sd

def audio_callback(indata, frames, time_info, status, bridge, jaw_channel=2):

amplitude = np.abs(indata).mean()

angle = 90 + min(amplitude * 400, 40)  # map amplitude to a jaw-open range

bridge.set_angle(jaw_channel, angle)

def run(bridge, samplerate=16000, blocksize=512):

with sd.InputStream(channels=1, samplerate=samplerate, blocksize=blocksize,

callback=lambda *a: audio_callback(*a, bridge=bridge)):

input("Streaming jaw sync from audio input — press Enter to stop.\n")

# 3. Microcontroller-side: Arduino/ESP32 servo receiver

Receives "SET <channel> <angle>" lines over serial and writes them to a PCA9685.

// servo_receiver.ino

#include <Wire.h>

#include <Adafruit_PWMServoDriver.h>

Adafruit_PWMServoDriver pwm = Adafruit_PWMServoDriver(0x40);

const int SERVO_MIN = 150;  // pulse length count out of 4096, calibrate per servo

const int SERVO_MAX = 600;

void setup() {

Serial.begin(115200);

pwm.begin();

pwm.setPWMFreq(50);  // standard analog servo frequency

}

int angleToPulse(float angle) {

return map((int)angle, 0, 180, SERVO_MIN, SERVO_MAX);

}

void loop() {

if (Serial.available()) {

String line = Serial.readStringUntil('\n');

int channel, angle;

if (sscanf(line.c_str(), "SET %d %d", &channel, &angle) == 2) {

if (channel >= 0 && channel < 16 && angle >= 0 && angle <= 180) {

pwm.setPWM(channel, 0, angleToPulse(angle));

}

}

}

}

# 4. Safety Watchdog (Python, Pi-side)

Halts all servo motion if no valid command loop / heartbeat has run recently, or if E-stop is triggered.

# safety_monitor.py

import time

import RPi.GPIO as GPIO

ESTOP_PIN = 17  # example GPIO pin wired to a physical E-stop switch

class SafetyMonitor:

def __init__(self, bridge, timeout_s=1.0):

self.bridge = bridge

self.timeout_s = timeout_s

self.last_heartbeat = time.time()

GPIO.setmode(GPIO.BCM)

GPIO.setup(ESTOP_PIN, GPIO.IN, pull_up_down=GPIO.PUD_UP)

def heartbeat(self):

self.last_heartbeat = time.time()

def check(self, neutral_pose):

estop_triggered = GPIO.input(ESTOP_PIN) == GPIO.LOW

stale = (time.time() - self.last_heartbeat) > self.timeout_s

if estop_triggered or stale:

self.bridge.set_pose(neutral_pose)

return False

return True

# Notes

- These skeletons assume a serial link between Pi and microcontroller; swap for I2C-direct-from-Pi if you drop the ESP32/Mega and drive the PCA9685 straight from the Pi (simpler, but loses the real-time isolation discussed in the Operating Software document).

- Calibrate SERVO_MIN/SERVO_MAX per physical servo — these pulse-count values vary by servo brand/model and should come from bench testing, not assumed.

- Log every pose change and safety event through the logger module referenced in Operating Software — see the Calibration Logs document for the format.