Animatronic Build — Equipment List

Full-size animatronic figure — Height: 5'7" (170 cm)

This list covers structure, motion, audio, control, power, and logging equipment sized for a 5'7" build. Quantities are estimates for a first prototype and should be adjusted after a working mockup of the head/arm subsystem. See the note on likeness and voice rights before sourcing scan or voice-cloning services.

# 1. Structural Skeleton

| Item | Spec / Notes | Qty |
| --- | --- | --- |
| Aluminum extrusion (spine) | 20x20mm T-slot, cut to ~54" (torso+neck run, leaves room for head/leg mounts) | 2-3 lengths |
| Aluminum tube/rod (limbs) | 12-16mm OD, cut per limb segment (upper arm ~11", forearm ~10") | 4 arms segments, 4 leg segments if legs included |
| T-slot corner brackets & gussets | For extrusion joints | 12-20 |
| Ball bearings (joint pivots) | 608ZZ or similar, sized to shaft | 10-14 |
| Threaded rod / bolts, various | M4-M8 mixed for joint hardware | Assorted pack |
| Base/stand mounting plate | Steel or aluminum plate, bolts to feet or a display stand | 1 |
| Neck gimbal bracket | 2-axis (pan/tilt) mount for head | 1 |

# 2. Arm & Hand Motion

Sized for a 5'7" figure: upper arm ~11 in, forearm ~10 in, hand ~7 in per side.

| Item | Spec / Notes | Qty |
| --- | --- | --- |
| Shoulder servo (pitch) | Digital servo or Dynamixel MX-64/XM430, high torque | 2 (1 per arm) |
| Shoulder servo (roll/abduction) | Same class as above | 2 |
| Elbow servo | Dynamixel XM430 or MG996R depending on load | 2 |
| Wrist rotation servo | Smaller digital servo, e.g. MG92B | 2 |
| Hand/finger actuation | Linear micro-servos or fishing-line tendon + micro-servo per finger group | 2 hands (5-10 servos each if per-finger) |
| Servo horns & linkages | Metal gear horns, matched to servo output | Assorted |
| Torque-rated joint bearings | For shoulder/elbow load bearing | 4-6 |

# 3. Head — Face, Mouth, Eyes

| Item | Spec / Notes | Qty |
| --- | --- | --- |
| Jaw hinge servo | MG996R or digital equivalent, mounted at jaw pivot | 1 |
| Audio-to-servo jaw driver board | Envelope-follower board (drives jaw from audio amplitude in real time) | 1 |
| Eye pan/tilt mechanism | 3D-printable animatronic eye assembly (common open-source design), 2 eyeballs on shared yoke | 1 set |
| Eye pan servo | Micro servo, e.g. SG90/MG90S | 1 |
| Eye tilt servo | Micro servo | 1 |
| Eyelid servos (blink) | Micro servo, upper lid at minimum | 2 (upper) + 2 (lower, optional) |
| Eyeball props | Acrylic or printed+painted eyeballs, sized to socket | 2 |
| Eyebrow/brow servos (optional, expression) | Micro servo for brow lift | 2 |
| Neck pan/tilt servos | Medium-high torque, supports head weight | 2 |

# 4. Speech & Audio

| Item | Spec / Notes | Qty |
| --- | --- | --- |
| Mini PC / Raspberry Pi 5 (8GB) | Runs TTS engine and high-level behavior logic | 1 |
| Class-D mini amplifier board | Matched to speaker impedance/wattage | 1 |
| Full-range speaker | 3-4in, chest-cavity mounted | 1-2 |
| Microphone (if interactive) | USB or I2S mic module, for triggering responses | 1 |
| TTS engine (software) | Off-the-shelf or licensed TTS voice — NOT a clone of a real person's voice without consent | License |

On voice: use a generic TTS voice, a hired voice actor, or an officially licensed voice. Cloning a real, identifiable person's voice without consent isn't something to build toward, even for a personal project.

# 5. Control Electronics

| Item | Spec / Notes | Qty |
| --- | --- | --- |
| Raspberry Pi 5 (or 4, 8GB) | Main brain: TTS, behavior logic, logging | 1 |
| Servo controller board | PCA9685 16-channel PWM driver (I2C) | 1-2 (stack for >16 servos) |
| ESP32 or Arduino Mega | Real-time servo timing / sensor polling, offloaded from Pi | 1 |
| Motor driver (if steppers used) | e.g. TMC2209 or A4988 per stepper | As needed |
| Wiring harness + connectors | JST/Dupont/servo extension leads, labeled by joint | Assorted |
| Status LEDs / indicator panel | Power/fault indication, optional external panel | 1 set |

# 6. Logging & Storage

| Item | Spec / Notes | Qty |
| --- | --- | --- |
| MicroSD card (Pi boot) | 64-128GB, high endurance rated | 1 |
| USB SSD or NVMe (via USB adapter) | For logs: servo states, audio triggers, errors, uptime | 1 (256GB+) |
| Logging software | SQLite or simple CSV logger, cron'd or event-driven | Software only |
| Wi-Fi module | Built into Pi; optional external antenna for better range inside the shell | 1 |
| Optional remote sync | Cloud bucket or home server push for offsite log backup | Software/service |

# 7. Power System

| Item | Spec / Notes | Qty |
| --- | --- | --- |
| Main power supply | 5V/6A+ for Pi/logic rail | 1 |
| Servo power supply (BEC or bench supply) | 6-7.4V, 10-20A depending on servo count/load, separate from logic rail | 1 |
| Shared ground bus | Common ground between logic and servo rails | 1 |
| In-line fuses / breakers | Sized per rail, protects against shorts/jams | 2-4 |
| Accessible E-stop switch | Physical cutoff, mounted for external access | 1 |
| Battery pack (if untethered) | LiPo or SLA, sized to runtime target, with BMS | 1 (optional) |

# 8. Shell & Finishing

| Item | Spec / Notes | Qty |
| --- | --- | --- |
| PETG or ASA filament | Structural shell panels (torso, limbs) | ~8-12kg for full body at this scale |
| Resin + resin printer access | Face and hands — fine detail areas | Face + 2 hands |
| Silicone skin (optional, face/hands) | Platinum-cure silicone for realistic texture over printed core | Kit, face+hands |
| Paint & finishing supplies | Primer, airbrush paints, sealant matched to filament/resin/silicone | Assorted |
| Wig / hair (if desired) | Synthetic or human-hair wig, styled to reference | 1 |
| Fasteners for panel access | Quarter-turn or magnetic panel latches for maintenance access | Assorted |

# Notes on Scale (5'7" / 170cm)

Approximate segment lengths to keep proportions correct at this height:

| Segment | Approx. length |
| --- | --- |
| Head (crown to chin) | ~9 in |
| Neck | ~4 in |
| Torso (shoulder to hip) | ~24 in |
| Upper arm | ~11 in |
| Forearm | ~10 in |
| Hand | ~7 in |
| Upper leg (if included) | ~17 in |
| Lower leg (if included) | ~17 in |
| Foot | ~9.5 in |

Reminder: if this figure depicts a real, identifiable public person and is intended for anything beyond private personal use — display, sale, publication, or public exhibition — obtain permission for likeness and voice use. Right-of-publicity rules apply to realistic reproductions like this.